PRILOG II + III : TEHNIČKE SPECIFIKACIJE + TEHNIČKA PONUDA/ ANNEX II + III: TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Naš broj/ Our ref. : SP/SUP-004-1

Predmet: Stožerno zapovjedno vozilo za službu zaštite i spašavanja...  za projekt.......	p 1 /… / str. 1/…

Subject: Mobile command vehicle for protection and rescue service...  for project...

Kolone 1 i 2 popunjava Ugovorno tijelo / Columns 1-2 should be completed by the contracting authority

Kolone 3 i 4 popunjava Ponuditelj/ Columns 3-4 should be completed by the tenderer

Kolona 5 je predviđena za ocjenjivačku komisiju/ Column 5 is reserved for the evaluation committee

Prilog III -  tehnička ponuda Ponuditelja/ Annex III - the contractor's technical offer

Ponuditelji trebaju popuniti obrazac na sljedećim stranicama:/ The tenderers are requested to complete the template on the next pages:

- Kolonu 2 popunjava Ugovno tijelo  u njoj se prikazuju tražene specifikacije (koje Ponuđač ne smije mijenjati),/ Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Kolonu 3 popunjava Ponuditelj i u njoj mora navesti pojedinosti svoje ponude (na primjer, riječi “usuglašeno” ili “da” nisu dovoljne)/ Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Kolona 4 omogućuje Ponuditelju unijeti svoje komentare na predloženu robu te da se eventualno referira  na određenu dokumentaciju/ Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

U eventualno dostavljenoj dokumentaciji trebaju biti jasno naznačeni (istaknuti, označeni) ponuđeni modeli i uključene opcije, ako ih ima, tako da ocjenjivači mogu vidjeti točnu konfiguraciju. Komisija za ocjenjivanje može odbaciti ponude koje ne omogućuju točnu identifikaciju modela i specifikacije.

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Broj stavke / Item number | 2. Opis tražene specifikacije | 3. Ponuđene specifikacije/ Specifications offered | 4. Bilješke, primjedbe, referenca na dokumentaciju/ Notes, remarks, ref to documentation | 5. Bilješke odbora za ocjenjivanje/ Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1. | STOŽERNO ZAPOVJEDNO VOZILO/ MOBILE COMMAND VEHICLE Količina 1/ Quantity 1 Tehnička specifikacija/ Technical specification Tip vozila: Novo stožerno zapovjedno vozilo, sa jednostrukom kabinom i odvojenim tovarnim prostorom, sa nadogradnjom i pratećom opremom, namijenjeno za operacije komuniciranja i planiranja vatrogasnih operacija na terenu/ Vehicle type: New mobile command vehicle, with single cabin and separate cargo space, converted and supplied with equipment, dedicated to communication and planning firefighting field operations Godina proizvodnje: minimum 2025. god– novo/ Year of production: minimum 2025 - new Snaga motora: min. 130kw/177 ks/ Engine power: min. 130 kw / 177 hp Zapremina motora: max. 2000 ccm/ Engine capacity: max. 2000 ccm Vrsta agregata: turbo diesel/ Turbocharger: turbo diesel Pogon vozila: 4x4/ Vehicle drive: 4x4 Mjenjač vozila: automatski/ Vehicle gearbox: automatic Pogonsko gorivo: euro dizel, vozilo mora biti opremljeno spremnikom sa AdBlue. Standard: EURO VI/ Fuel: euro diesel, the vehicle must be equipped with an AdBlue tank. Standard: EURO VI Dimenzije vozila:/ Vehicle dimensions: - min. dužina vozila 6800 mm/ min. vehicle length 6800 mm - maks. širina vozila 2100 mm/ max. vehicle width 2100 mm - međuosovinski razmak min. 4400 mm/ wheelbase min.4400 mm - visina prostora za putnike min. 1950 mm/ passenger compartment height min. 1950 mm Nosivost vozila: max.3500 kg/ Payload : max.3500 kg Broj sjedišta unutar kabine: min. 1+2 / Number of seats in the cabin: min. 1 + 2 Broj i raspored vrata: min. 5, naprijed lijevo i desno, klizna vrata s desne strane vozila, zadnja dvokrilna vrata bez prozora / Number and placement of doors: min. 5, front left and right, right side sliding doors, rear double doors without windows Centralno zaključavanje sa min 2 ključa/ Central lock system with min. 2 keys Oprema vozila:/ Vehicle equipment: - vozilo mora posjedovati kuku za vuču /the vehicle must have a towing eye - klima unutar kabine vozila: manualna / Air conditioning: manual, - zaključavanje: centralno/ Locking: central - zračni jastuci min. za vozača I suvozača, / Airbags min. driver’s and codriver’s - tempomat/ Cruise control, - sustav upravljanja - hidraulični / Control system - hydraulic, - Tvornički radio uređaj sa min. 10” ekranom u boji i osjetljivim na dodir, interface za mobitel sa induktivnim punjenjem, min. 4 zvučnika/ Radio device with min. 10" color and touch screen, mobile phone interface with inductive charging, min. 4 speakers, - Pomoć pri parkiranju (parking senzori): naprijed i nazad / Parking assist (parking sensors): front and rear - Kamera za nadzor okoline odstraga/ Rear camera - Vanjski retrovizori: elektronsko podesivi, sklopivi i grijani / Exterior mirrors: electronically adjustable, retractable and heated, - Električni podizači stakala naprijed/electric front windows, - Pomoć pri kretanju na uzbrdici/ Hill hold assist, - Sistem protiv blokiranja točkova ABS / ABS anti-lock brake system, - Elektronska kontrola stabilnosti ESP / Electronic stability control ESP, - Start Stop sustav/Start Stop system - Sustav podrške za vožnju unutar vozne trake/ Lane assist safety system - Sustav nadzora okoline i funkcija kočenja u nuždi/ Environmental sensor system and automatic distance control - Digitalni kokpit/ Digital cockpit - Dnevna svjetla/ Daily lights - Automatska kontrola glavnih svjetala/ Automatic headlight control - Svjetla za maglu/ Fog lights - Min. 2 utičnice 12V/ Min.2 12V sockets - Gume: M + S/ Tires: M + S - Rezervni kotač, alat i dizalica/ Spare wheel with tool and car jack - Sustav nadzora tlaka u gumama/ Tire Pressure Monitoring System - Set dodatne opreme: kutija prve pomoći, trokut, reflektirajući prsluk/ Set of additional equipment: first aid kit, triangle, reflective vest -Boja vozila mora biti crvena RAL 3000, sa oznakama i natpisima Naručitelja na vratima. Na haubi i na zadnjem dijelu vozila natpis "VATROGASCI" izrađen od bijele reflektirajuće trake. /The colour of the vehicle must be red RAL 3000, with the markings and inscriptions of the Client on the door. On the hood and at the rear of the vehicle the inscription "FIREFIGHTERS" must be made of white retroreflective tape. DORADA VOZILA: Vanjski dio - Na krovu kabine vozila mora biti ugrađena plava LED svjetlosna signalizacija, min. 1140 mm duljine, upravljiva putem kontrolne jedinice smještene u kabini vozila, sa integriranim zvučnikom snage min. 100W /A blue LED light signal lightbar must be installed on the roof of the cabin, with a length of min. 1140mm, with integrated speakerphone of min. 100W power, controlled from the cockpit via control device. - Na prednjoj i zadnjoj strani vozila, moraju biti instalirana plava trepćuća svijetla (bljeskalice) kojima se upravlja preko upravljačke jedinice smještene u kabini vozila. / Blue flashing lights (blinkers) must be installed on the front and rear bumper, controlled via control unit, placed in the cockpit. - Unutrašnjost prtljažnog dijela vozila mora biti podijeljena na dva odjeljka – jedan za smještaj uredske mobilne jedinice sa min. 2 radna mjesta i drugi za smještaj opreme za komunikaciju i druge opreme/ The interior of the cargo space must be divided in two parts- one for the placement of the office mobile unit with min. of 2 working places and the second one for accommodation of communication and miscellaneous equipment. Odjeljak za smještaj uredske mobilne jedinice/ Mobile office unit section: Pod prostora mora biti izrađen od protuklizne vodonepropusne šperloče, debljine min. 15mm, dodatno presvučen protukliznom pvc obogom, otpornom na habanje, visoke otpornosti na primjenu dezinfekcijskih sredstava za čišćenje, sa završnim aluminijskim lajsnama na svim potrebnim pozicijama/ The floor has to be made of anti-slip waterproof plywood, min. 15mm thick, additionally coated with anti-slip PVC covering, resistant to wear, highly resistant to the use of disinfectant cleaning agents, finalized with aluminium skirts in all necessary positions. Zidovi furgona moraju biti obloženi zvučno-toplotnom izolacijom cijelom površinom, te obloženi posebnom ABS oblogom u skladu sa interijerom vozila, sa posebno tapaciranom krovnom oblogom. Donji dijelovi zidova moraju biti obloženi aluminijskim limom debljine min 2mm, za dodatnu zaštitu od habanja i lakše održavanje/ Complete wall surface of the van must be isolated against sound and heat influence and additionally covered with a special ABS trims, in compliance to the vehicle interior, with a specially roof upholstery. The lower parts of the walls must be covered with aluminium sheet min. 2mm thickness, for additional protection against wear and tear and easier maintenance. Radni stolovi za operatere – radne plohe, za smještaj računara i opreme moraju biti postavljeni i čvrsto montirani na zidove, izrađeni od šperploče min 35mm debljine, sa zaobljenim prednjim uzdužnim rubom, laminirano i otporno na vlagu / Desks for operators- working surfaces must be suitable for the placement of computers and equipment, constructed and firmly mounted on walls, made of plywood min. 35mm thick, with a rounded front longitudinal edge, laminated and moisture-resistant, Min. 2 ormarića sa 3 ladice sa bravama za zaključavanje, moraju biti smještene u podnožju radnih ploha, na način da ne ometaju sjedenje i osiguravaju jednostavan i izravan pristup sadržaju/ There must be min. 2 cabinets with 3 drawers with locks, placed at the base of the worktops, in such a way to enable sitting and ensure easy access to the content. Min. 2 ergonomski oblikovana sjedala, sa mogućnošću podešavanja nivoa naslona, sa rukohvatima i izvlačivim osloncem za noge, moraju biti fiksno pričvršćena u pod vozila/ Min. 2 ergonomically shaped chairs with rectangular seatback and foot rest, must be fixed to the floor of the vehicle. Duljinom zida, na min. 2 strane,moraju biti postavljene čvrste vodilice/šine za prihvat nosača ekrana/ There must be solid brackets – raills, mounted along the length of the wall, on at least 2 sides, installed to accommodate the screens. Duljinom zida postavljene vodilice unutar kojih je provedena električna instalacija sa utičnicama, min. 10 kom, za napajanje elektroničke opreme./ Guides installed along the length of the wall, with electrical installation inside, with sockets, min. 10 pcs, is for electic power supply Duljinom cijelog stropa mora biti postavljena LED rasvjeta/ along the ceiling there must be mounted LED lights Električna uvlačiva stepenica mora biti montirana na ulazna bočna vrata, min. kapaciteta nosivosti 250kg, sa površinom gazišta min. 750mm/ automatic sliding step must be mounted at the side doors, min. capacity of 250kg, min. compression surface width 750mm. Stražnji prag vozila mora biti ojačan pocinčanim čelikom na način da može biti korišten kao oslonac pri ulasku/ Ther rear door step must be reinforced with stainless steel in such a way as to be used as an support for entrance. Stražnja dupla vrata sa obje unutarnje strane moraju biti zaštićena rebrastim aluminijskim limom/ Rear double doors on booth sides must be protected with corrugated aluminum sheets. Grijanje i hlađenje prostora uredske mobilne jedinice/Mobile office unit heating and cooling - U odjeljku za smještaj uredske mobilne jedinice mora postojati autonomni uređaj za rashlađivanje kapaciteta min.2700W, sa krovnom jedinicom, sa difuzorima zraka postavljenim na način da osiguravaju ravnomjerno i neometano hlađenje prostora/ Mobile office unit section must be equipped with autonomous cooling device, capacity min. 2700W, with air diffusers placed in such a way as to ensure uniform and uninterrupted cooling - U odjeljku za smještaj uredske mobilne jedinice mora postojati autonomni uređaj za grijanje, kapaciteta min. 2000 W / Mobile office unit section must be equipped with autonomous heating device, capacity min.2000 W Odjeljak za smještaj opreme za komunikaciju i druge opreme/ Communication and various equipment compartment - Odjeljak za smještaj opreme za komunikaciju i druge opreme mora biti smješten u krajnjem zadnjem dijelu tovarnog prostora, potpuno odvojen od uredskog dijela vozila. Između uredskog i odjela za smještaj opreme mora postojati pregrada, koja vizualno i zvučno odvaja dva prostora. Unutar prostora za pohranu opreme mora postojati min. 5 polica unutar zatvorenog ormara sa bravom za zaključavanje, napravljenih od čvrstog materijala, otpornog na prohrđavanje,sa izvlačivim vodilicama, gdje se oprema može pohraniti na način da je osigurana za vrijeme vožnje i da joj je omogućen brz i lagan pristup/ Communication and various equipment compartment must be positioned in the very rear part of the cargo space, completely separated from the office part of the vehicle. Between office and equipment compartment must be separation wall that visually and audibly separates two parts. Inside equipment compartment there must be min.5 shelfs within closed locker, made of firm material rust proof and with retractable rails, where the equipment can be preserved in a way that is safely secured during the transport but at the same time can be easily and conveniently reached. - Zidovi odjeljka moraju biti obloženi aluminijskim rebrastim limom, debljine min. 2/3mm./ Compartment walls must be coated with corrugated aluminum, min 2/3 mm tick. - Elektro sustav/ Electro system - unutar vozila mora biti provedena niskonaponska mreža 220V/ A 220 low voltage network must be installed inside the vehicle - Mora postojati mogućnost napajanja vozila iz vanjskog izvora/ There must be an option of power supply from an external source. - Izvedba električne mreže mora biti takva da osigurava neometan rad trošila i ugrađene opreme/ Electrical network must be designed in such a way as to ensure uninterrupted work of appliances and installed devices. - Duljinom zidova unutar ATEST VOZILA -Atest vozila: Atest vozila za nadogradnju i opremu/ Vehicle certificate: Vehicle certificate for upgrades and equipment OPREMA/ EQUIPMENT: - AGREGAT za struju inverter, snage min. 3,5 kW, nivoa buke max. 70 dB smješten na izvlačivoj polici,/ AGGREGATE for power inverter, power min. 3.5 kW, noise level max. 70 dB located on a pull-out shelf, - 230 V inverter - Prijenosni sustav za napajanje s LiFePO₄ baterijom, kapaciteta većeg od 2000Wh i sa sklopivim 400 W monokristalnim solarnim panelom s IP68 zaštitom, pogodan za mobilnu i stacionarnu upotrebu Kapacitet: 2048Wh Snaga: 2400W X-Boost: 3100W Utičnice: 4x AC utičnice Maksimalni solarni ulaz: 1000W USB-A Izlazi: 2x, 5V, 2.4A, 12W Max USB-A Fast Charge Izlazi: 2x, 5V, 2.4A / 9V, 2A / 12V, 15A, 18W Max USB-C Izlazi: 2x, 5/9/12/15/20V, 5A, 100W Max DC5521 Izlaz: 2x, 12.6V, 3A, 38W Max AC Punjenje: X-stream Fast Charge 2300W, 10A Solarno Punjenje:11-60V, 15A, jedan konektor 500W; dva konektora 1000W Bluetooth Wi-fi Težina (kg): 23 Sklopivi 400 W monokristalni solarni panel Jamstvo: 60 mj Ovaj sustave za napajanje namjenjenn je i za napajanje prijenosnog stupa - Portable power system with LiFePO₄ battery, capacity greater than 2000Wh and with foldable 400W monocrystalline solar panel with IP68 protection, suitable for mobile and stationary use Capacity: 2048Wh Power: 2400W X-Boost: 3100W Outlets: 4x AC outlets Maximum solar input: 1000W USB-A Outputs: 2x, 5V, 2.4A, 12W Max USB-A Fast Charge Outputs: 2x, 5V, 2.4A / 9V, 2A / 12V, 15A, 18W Max USB-C Outputs: 2x, 5/9/12/15/20V, 5A, 100W Max DC5521 Output: 2x, 12.6V, 3A, 38W Max AC Charging: X-stream Fast Charge 2300W, 10A Solar Charging: 11-60V, 15A, one connector 500W; two connectors 1000W Bluetooth Wi-fi Weight (kg): 23 Foldable 400 W monocrystalline solar panel Warranty: 60 months This power system is also intended for powering a portable pole INFORMATIČKA OPREMA: -RAČUNALA -2 Računala - Procesor 6 jezgri s hladnjakom rada do 4,60 GHZ Matična ploča podržava rad novijih procesora, Ram memorije DDR4, podržava grafičke karte GDDR6 memorije, RAM 16 GB DDR4 3200MHZ, SSD 1TB M2, Grafička kartica memorije 8 GB tip memorije GDDR6, širina sabirnice 128bit, naponski kabel 8-pin, Napojna jedinica 750 W Kućište, Windows 11 PRO licenca/ Office 2021 Pro licenca/Computer - 6-core processor with cooler working up to 4.60 GHZ Motherboard supports newer processors, DDR4 RAM, supports GDDR6 graphics cards, RAM 16 GB DDR4 3200MHZ, SSD 1TB M2, Graphics card memory 8 GB GDDR6 memory type, bus width 128bit, power cable 8-pin, Power supply unit 750 W Case, Windows 11 PRO license/ Office 2021 Pro license. -MONITOR: Veličina: 23,8” 4 MONTORA Tip: IPS Rezolucija: 1920×1080 FHD Refresh rate: 180Hz Odziv: 1ms Omjer: 16:9 Kontrast: 1000:1 Kut vidljivosti: 178°/178° Osvjetljenje: 250cd/m2 Konektori: 2x HDMI, DisplayPort VESA: 75 x 75 Potrošnja: 22W FreeSync Premium HDR10/ -PISAČ (Kolor/ Laserski) Namjena uređaja: stolni pisač za ispis u boji Tehnologija ispisa: laserska tehnologija u boji Maksimalna rezolucija ispisa: 1200 x 1200 dpi. Radna memorija: minimalno 1 GBBrzina ispisa: najmanje 18 stranica u minuti (A4, jednostrano). Mogućnost ispisa s više uređaja putem žične i bežične mreže Načini povezivanja: – Ethernet mrežna veza (LAN) – Bežična Wi-Fi veza – USB veza visoke brzine Podržani operacijski sustavi: Windows, macOS, Linux Ulazni kapacitet papira: minimalno 250 listova u glavnu ladicu + 1 višefunkcijski ulaz Vrsta podržanih medija: običan papir, deblji papir, prozirne folije, naljepnice, omotnice Mogućnost mobilnog ispisa putem standardiziranih mobilnih protokola (npr. Mopria, AirPrint) Upravljanje uređajem putem integriranog LCD zaslona (u boji ili monokromatskog) Uređaj mora imati mogućnost rada u mrežnim okruženjima te omogućiti jednostavno upravljanje i konfiguraciju putem internetskog sučelja Uređaj mora biti isporučen s pripadajućim početnim potrošnim materijalom -TIPKOVNICA + MIŠ BEŽIČNI Tipkovnica i miš wireless 2.4 GHz, USB nano prijemnik , domet 10 metara, napajanje 2 x AAA baterija ( tipkovnica ), 2 x AA baterija (miš). Tipkovnica sa numeričkim tipkama, miš optički, 3 tipke/ KEYBOARD + MOUSE WIRELESS Keyboard and mouse wireless 2.4 GHz, USB nano receiver, range 10 meters, power supply 2 x AAA batteries (keyboard), 2 x AA batteries (mouse) Keyboard with numeric keys, optical mouse, 3 buttons -SLUŠALICE: Over-ear (preko uha), gaming Mikrofon: Unidirekcioni, odvojivi Konekcija: Bežična, 2.4 GHz USB Frekventni opseg: 10 Hz – 20.4 kHz/ HEADPHONES: Over-ear, gaming Microphone: Unidirectional, detachable Connection: Wireless, 2.4 GHz USB Frequency range: 10 Hz – 20.4 kHz -ALAT I PRIBOR Za održavanje je priložen sljedeći alat: Šasija: dizalica sa pripadajućim polugama, montažna poluga, torba odnosno kofer s alatom -TOOLS AND ACCESSORIES The following tools are included for maintenance: Chassis: jack with associated levers, assembly lever, tool bag or case DOKUMENTACIJA (hrvatski jezik??) Upute za uporabu vozila s specifikacijom, servisna knjižica (DA, na hrvatskom zu isporučeno vozilo), Upute za upotrebu svjetlosno zvučne signalizacije (DA, na hrvatskom zu isporučeno vozilo)/DOCUMENTATION (Croatian language??) Vehicle operating instructions with specification, service booklet (YES, in Croatian for the delivered vehicle), Instructions for using the light and sound signalling (YES, in Croatian for the delivered vehicle)/ OPĆI ZAHTJEVI SIGURNOSTI: Materijali od kojih je izrađeno vozilo moraju biti nezagađujući i ne smiju uticati na zdravlje korisnika/ GENERAL SAFETY REQUIREMENTS: The materials of which the vehicle is made must be non-polluting and must not affect the health of the user SERVIS, GARANCIJE, ROKOVI ISPORUKE Ovlašteni servis: DA/ Authorized service: YES GARANCIJA: min. 24 mjeseca na vozilo, ugrađenu opremu, nadogradnju i isporučenu opremu WARRANTY: min. 24 months on the vehicle, installed equipment, conversion and delivered equipment Rok za osiguranje rezervnih dijelova nakon prestanka proizvodnje modela vozila; najmanje 7 godina nakon prestanka proizvodnje / Term for the insurance of spare parts after the end of production of the vehicle model; at least 7 years after the end of production Izjava o tipskoj sukladnosti vozila; DA / Declaration of vehicle type conformity; YES ROK ISPORUKE: Najdulje 90 dana od potpisa ugovora (obavezan zahtjev i ne boduje se). / DELIVERY DEADLINE: Maximum 90 days from the signing of the contract (mandatory requirement and not scored). MOBILNI STUP Mobilni stup opremljen optičkom i termalnom kamerom na gimbalu, meteorološkom stanicom te pratećom infrastrukturom potrebnom za njegovu funkcionalnu upotrebu. Sustav je namijenjen za terensko motrenje i nadzor te omogućuje prikupljanje i prijenos video i meteoroloških podataka u stvarnom vremenu. Mobilnost sustava omogućava brzo premještanje i postavljanje na različitim lokacijama, ovisno o operativnim potrebama korisnika. Stup mora biti izdržljiv, jednostavan za transport i postavljanje, te prilagođen za rad u različitim vremenskim uvjetima. Uključena je i tehnička oprema za napajanje i komunikaciju, kao i potrebni nosači i kućišta za stabilan i siguran rad sustava na terenu. Mobile mast equipped with optical and thermal camera mounted on a gimbal, meteorological station, and supporting infrastructure required for its functional operation. The system is intended for field surveillance and monitoring, enabling real-time collection and transmission of video and meteorological data. Its mobility allows for rapid relocation and deployment at various sites, depending on the operational needs of the user. The mast must be durable, easy to transport and set up, and designed for operation under various weather conditions. Included are the necessary power supply and communication components, as well as mounts and housings required to ensure stable and secure field operation of the system. |  |  |  |
|  | OPTIČKA I TERMALNA KAMERA NA GIMBALU Fizičke karakteristike Masa: manja od 1000g Dimenzije (DxŠxV): manje od 180×150×160 Ulazno napajanje: 14.5 – 52 V DC Potrošnja: prosjek max 25 W, vršna max 60 W (max 5 @12 V) Temperatura rada: –10 °C do +50 °C; skladištenje: –20 °C do +50 °C Gimbal Stabilizacija: ±0.01° vibracijski raspon Upravljanje: modes Lock, Follow, Off Opseg pokreta: Opseg pokreta: Tilt ±120° (mehanički ±143°) Pan ±310° (mehan. ±320°) Roll ±45° (mehan. -260° do +80°) Optička kamera Senzor: CMOS sensor min 8 MP i optički zoom do 20x ili 50MP kamera bez zooma Termalna kamera Senzor: 640×512 VOx mikrobolometar; 12 µm pixel Termalno područje: 8–14 µm; NETD ≤ 50 mK Sučelja i programska podrška Sučelja: UART, Ethernet, SBUS Hardver autopilota: Pixhawk, CubePilot, Skynode; softver: ArduPilot, PX4 Kontroleri: QGroundControl i Gremsy Payload Assistant app SDK: Gremsy Payload SDK Streaming: HDMI, RTSP Značajke: RTSP video, geotagging, pinpoint, AI objekt detekcija (ljudi, vozila), PiP prikaz EO/IR, praćenje 30fps Jamstvo na proizvod minimalno 1 godina PONUĐENI MODEL (upisati) OPTICAL AND THERMAL CAMERA ON A GIMBAL Physical characteristics Weight: less than 1000g Dimensions (LxWxH): less than 180×150×160 Input power supply: 14.5–52V DC Consumption: average max 25W, peak max 60W (max 5 @12V) Operating temperature: –10°C to +50°C; storage: –20°C to +50°C Gimbal Stabilization: ±0.01° vibration range Management: modes Lock, Follow, Off Range of motion: Range of motion: Tilt ±120° (mechanical ±143°) Pan ±310° (mechanical ±320°) Roll ±45° (mechanical -260° to +80°) Optical camera Sensor: CMOS sensor min 8MP and optical zoom up to 20x or 50MP camera without zoom Thermal camera Sensor: 640×512 VOx microbolometer; 12µm pixel Thermal range: 8–14µm; NETD ≤ 50mK Interfaces and software support Interfaces: UART, Ethernet, SBUS Autopilot hardware: Pixhawk, CubePilot, Skynode; software: ArduPilot, PX4 Controllers: QGroundControl and Gremsy Payload Assistant app SDK: Gremsy Payload SDK Streaming: HDMI, RTSP Features: RTSP video, geotagging, pinpoint, AI object detection (people, vehicles), PiP display EO/IR, 30fps tracking Minimum 1 year product warranty MODEL OFFERED (please specify) |  |  |  |
|  | Automatska meteorološka stanica s pripadajućom opremom Vanjska senzorska jedinica (ISS – Integrated Sensor Suite) Temperatura zraka: -40 °C do +65 °C, preciznost ±0.5 °C Vlažnost zraka: 0 – 100 %, preciznost ±3 % Tlak zraka: 540 – 1100 hPa, preciznost ±1.0 hPa Brzina vjetra. 0.5 – 290 km/h, preciznost ±2 km/h Smjer vjetra: 0 – 360°, rezolucija 1°, preciznost ±3° Oborine: 0.2 mm rezolucija, kantica za kišu s kolektorom Napajanje: Solarno + backup baterija Komunikacija: Bežični prijenos (do 300 m u otvorenom prostoru) Kućište: Otporno na UV zračenje, vodootporno, montaža na stup PONUĐENI MODEL (upisati)/ Automatic weather station with associated equipment External sensor unit (ISS – Integrated Sensor Suite) Air temperature: -40 °C to +65 °C, accuracy ±0.5 °C Air humidity: 0 – 100 %, accuracy ±3 % Air pressure: 540 – 1100 hPa, accuracy ±1.0 hPa Wind speed. 0.5 – 290 km/h, accuracy ±2 km/h Wind direction: 0 – 360°, resolution 1°, accuracy ±3° Precipitation: 0.2 mm resolution, rain bucket with collector Power supply: Solar + backup battery Communication: Wireless transmission (up to 300 m in open space) Housing: UV-resistant, waterproof, pole-mounted MODEL OFFERED ( please specify) Data Logger Spremanje podataka: Interna memorija za > 2500 uzoraka Softver: PC aplikacija za preuzimanje i grafički prikaz podataka Izvoz podataka: CSV, TXT, ili kompatibilno za MS Exce Prijenos: Putem COM porta (serijski), moguća USB konverzija Interval zapisa: Konfigurabilno (min. 1 min) PONUĐENI MODEL (upisati) Data Logger Data storage: Internal memory for > 2500 samples Software: PC application for data download and graphical display Data export: CSV, TXT, or compatible with MS Exce Transfer: Via COM port (serial), USB conversion possible Recording interval: Configurable (min. 1 min) MODEL OFFERED ( please specify ) |  |  |  |
|  | Bežični predajnik Funkcija: Prijem podataka s vanjske ISS jedinice Komunikacija: Bežični prijem podataka s ISS jedinice Napajanje: 230 V AC adapter ili baterijsko napajanje Sučelje:Priključak za WeatherLink datalogger Jamstvo na proizvod minimalno 1 godina PONUĐENI MODEL (upisati)/ Wireless transmitter Function: Receive data from external ISS unit Communication: Wirelessly receive data from ISS unit Power supply: 230 V AC adapter or battery power Interface: Connection for WeatherLink datalogger Product warranty minimum 1 year MODEL OFFERED (( please specify ) |  |  |  |
| 3. | Teleskopski stup za montažu kamera i senzora s pripadajućom opremom Tehničke specifikacije stup Visina (maks.): 6 metara (teleskopski razvijen) Sklopiva dužina: ≤ 1.5 m u sklopljenom stanju Broj segmenata: Min. 5 – 6 segmenata (teleskopski) Materijal: Visokomodulna 3K karbonska vlakna Otpornost na savijanje: Visoka (namijenjena za vanjske uvjete i terensku montažu) Masa stupa: ≤ 3.0 kg Maksimalno opterećenje: Min. 3 kg na vrhu u potpuno razvijenom stanju Vrsta montaže: Samostojeći uz osiguranje s užadima i klinovima Otpornost na UV: Da Otpornost na vremenske uvjet: Kiša, sunce, vjetar (do 60 km/h uz stabilizaciju) Dodatna oprema za montažu i stabilizaciju Najlonska užad (guy line) dužine 10 m, promjer 4 mm, otporna na UV i habanje - kom3 Uzemljeni klinovi čelični, dimenzije 9 × 300 mm, pocinčani ili inox - kom 3 Gornji konektor: mehanički spojni dio za montažu kamere ili senzora (standardni navoj ili nosač po narudžbi) Jamstvo na proizvod minimalno 1 godina PONUĐENI MODEL (upisati)/ Telescopic pole for mounting cameras and sensors with associated equipment Technical specifications of the pole Height (max.): 6 meters (telescopically extended) Foldable length: ≤ 1.5 m when folded Number of segments: Min. 5 – 6 segments (telescopic) Material: High modulus 3K carbon fiber Bending resistance: High (intended for outdoor conditions and field installation) Mass of the pole: ≤ 3.0 kg Maximum load: Min. 3 kg at the top when fully extended Mounting type: Freestanding with rope and peg protection UV resistance: Yes Weather resistance: Rain, sun, wind (up to 60 km/h with stabilization) Additional equipment for mounting and stabilization Nylon ropes (guy line) 10 m long, diameter 4 mm, UV and abrasion resistant - pcs 3 Steel grounding pegs, dimensions 9 × 300 mm, galvanized or stainless steel - pcs 3 Upper connector: mechanical connecting part for mounting a camera or sensor (standard thread or custom mount) Minimum 1 year product warranty MODEL OFFERED ( please specify ) |  |  |  |
|  | Obradni računalni podsustav na rubu (edge computing subsystem) System-on-Module (SOM) niske potrošnje za upravljanje gimbalom, kamerom i meteo stanicom. Osigurava prijenos podataka sa senzora, kamera i meteo stanice do rutera u realnom vremenu prema standardnim protokolima. Za prijenos videa koristiti RTSP protokol i prijenos pune rezolucije u realnom vremenu do zapovjednog vozila ukoliko je fiksna povezanost ili bežično ukoliko je udaljenost manja od 10m. PONUĐENI MODEL (upisati) Edge computing subsystem Low-power System-on-Module (SOM) for controlling gimbal, camera and weather station. Provides real-time data transfer from sensors, cameras and weather station to router according to standard protocols. For video transmission, use RTSP protocol and real-time full resolution transmission to command vehicle if fixed connection or wireless if distance is less than 10m. MODEL OFFERED ( please specify ) |  |  |  |
|  | Ruter "Kompaktni SOHO ruter s ugrađenim CAT6 LTE modemom, 2.4 GHz Wi‑Fi 6, četiri Gigabit Ethernet porta, 256 MB RAM‑a i 128 MB NAND memorije, napaja se putem USB‑C (5 V/12 W), radi na RouterOS v7 (licenca nivo 4), bez ventilatora (pasivno hlađenje), radne temperature −40 °C do +60 °C, IP20 – predviđen za unutrašnju uporabu, dolazi s europskom i međunarodnom LTE podrškom te kompletom uključene opreme (napajanje, modem, antena) Jamstvo 1 godina" PONUĐENI MODEL (upisati) Router "Compact SOHO router with built-in CAT6 LTE modem, 2.4GHz Wi-Fi6, four Gigabit Ethernet ports, 256MB RAM and 128MB NAND memory, powered via USB-C (5V/12W), runs on RouterOSv7 (level 4 license), fanless (passive cooling), operating temperatures −40°C to +60°C, IP20 – intended for indoor use, comes with European and international LTE support and a set of included equipment (power supply, modem, antenna) Warranty 1 year" MODEL OFFERED ( please specify ) |  |  |  |
|  | Softver Klijentski softver za upravljanje kamerama i meteo stanicom te prihvat, obradu, pohranu i prikaz podataka. Softver mora preko grafičkog sučelja upravljati sustavom i osigurati prikaz svih relevantni podataka o sustavu. Sustav mora u realnom vremenu preuzimati podatke sa senzora (kamera i meteo stanice) sukladno korisničkim postavkama, prikazati ih korisniku, po potrebi spremati te ukoliko postoji veza slati iste u zapovjedni centar. PONUĐENI MODEL (upisati)/ Software Client software for controlling cameras and weather stations and receiving, processing, storing and displaying data. The software must manage the system via a graphical interface and ensure the display of all relevant system data. The system must download data from sensors (cameras and weather stations) in real time according to user settings, display them to the user, save them if necessary and, if there is a connection, send them to the command center. MODEL OFFERED ( please specify ) |  |  |  |

Napomena: Jedinična cijena stavke za ponuđeno vozilo mora sadržavati sve posredne i neposredne troškove, popuste i sva davanja sukladno Dokumentaciji o nabavi. Cijena se iskazuje zaokružena na dvije decimale. Ponuditelj mora ispuniti sve tražene podatke iz Tehničke specifikacije.

Note: The unit price of the item for the offered vehicle must include all indirect and direct costs, discounts and all charges in accordance with the Procurement Documentation. The price is expressed rounded to two decimal places. The Bidder must fill in all the required data from the Technical Specification.